Step-by-step guide for installing LED lights With a three 3-pin rocker switches, integrating a fuse tap and an additional circuit fuse board.

Table Of Contents

Step-by-step guide for installing LED lights	1

Components Needed:	1

Steps to Follow:	1

Step 1: Preparation	1

Step 2: Wiring the Rocker Switches	2

Step 3: Installing the Additional Fuse Board	2

Step 4: Connecting the LEDs to the Fuse Board	2

Step 5: Soldering and Securing Connections	3

Step 6: Testing	3

Step 7: Final Touches	3

## Components Needed:

1x Fuse Tap: For safely tapping into an existing fuse in your vehicle.

3x Rocker Switches (3-pin): These will control the LED lights.

1x Roll of Black Wire: For ground connections.

1x Roll of Red Wire: For power connections.

1x Wire Strippers: For preparing the wires.

1x Soldering Iron & Solder: To ensure secure and durable wire connections.

1x Circuit Fuse Board: An additional fuse board to organize power distribution and fuse protection.

## Steps to Follow:

### Step 1: Preparation

- Disconnect the vehicle's battery: This prevents any accidental short circuits or power surges.

- Choose the mounting location: Decide where to install the 3-pin rocker switches and additional fuse board (usually in an accessible area).

- Measure wire lengths: Ensure your red and black wires are long enough to reach between the LED lights, switches, and fuse board.

### Step 2: Wiring the Rocker Switches

A 3-pin rocker switch typically has:

- Pin 1 (Ground/Negative): Connect to a ground (black wire).

- Pin 2 (Load/Output): Connect to the LED lights.

- Pin 3 (Power/Input): Connect to a power source (red wire).

- Mount the rocker switches: Secure them in their desired location.

- Connect the ground (Pin 1):

  - Use black wire to connect all the rocker switches' Pin 1 to the vehicle chassis or a reliable ground point.

  - You can use a daisy chain setup, running one ground wire across all switches.

- Connect the load (Pin 2):

  - Use red wire to connect each rocker switch’s Pin 2 to the positive terminal of the LED lights. Each switch should correspond to one set of LED lights.

- Connect the power (Pin 3):

  - Each switch needs to be connected to the fuse board. Use red wires to connect Pin 3 of each rocker switch to a separate fused slot on the circuit fuse board.

### Step 3: Installing the Additional Fuse Board

- Mount the fuse board in an accessible location (e.g., under the dashboard or near the battery).

- Power input to the fuse board:

  - Use the fuse tap to draw power from an existing fuse in the vehicle's fuse box.

  - The fuse tap will have two connections: one to the fuse box and one to your new circuit fuse board.

  - Run a red wire from the output of the fuse tap to the input terminal of the new circuit fuse board.

- Ground the fuse board:

  - Connect the ground terminal of the fuse board to the vehicle chassis using black wire.

### Step 4: Connecting the LEDs to the Fuse Board

- Run power to the LEDs: For each LED light, connect the positive terminal to the load terminal (Pin 2) on its corresponding rocker switch.

- Ground the LEDs: Connect the negative terminal of each LED light to a ground point (chassis) using black wire.

### Step 5: Soldering and Securing Connections

- Strip the wires using your wire strippers before making connections.

- Solder all wire connections to ensure they’re secure and durable, especially for the switch pins and LED connections.

- Use heat shrink or electrical tape to insulate any exposed wires.

### Step 6: Testing

- Reconnect the battery and test each rocker switch to ensure the LEDs turn on and off properly.

- Ensure the fuse tap provides power when the vehicle ignition is on if you’re using an ignition-controlled fuse.

### Step 7: Final Touches

- Bundle and secure the wires: Use zip ties or loom tubing to clean up the wiring and prevent tangling.

- Check all connections: Ensure everything is tight and insulated properly to avoid short circuits.